MC Control board program update steps

一、安装MDK 5 Installing software MDK5

- 双击MDK529.exe  Double-click MDK529.exe

- 一直Next安装，直到软件安装完成 Continue Next installation until the software installation is complete

- 安装STM32 Pack包：打开Keil 5 按以下步骤打开安装窗口 1->2（File)弹出下拉菜单选择Import... -> 选择“Keil.STM32F0xx_DFP.1.5.0.pack”进行安装

Install STM32 Pack: open Keil 5. Follow the following steps to open the installation window 1->2 (File). -> select "Keil. Stm32f0xx_dfp.1.5.0.pack" to install

- J-Link驱动安装 J-link driver installation

双击Setup_JLink_V502d.exe进行J-link驱动安装 Double-click Setup JLink V502D. exe to install the J-Link driver

- 硬件连接 Hardware connection

- J-LINK SWD口连接，J-LINK是标准的ARM 20Pin JTAG端口，使用SWD口需要进行更改，JTAG接口定义如下

J-link SWD port connection. J-link is the standard ARM 20Pin JTAG port, which needs to be changed when using SWD port

Pin10  SWCLK

Pin8  SWDIO

Pin9 GND

Pin2 VCC_OUT

Pin1 VCC

- 将JTAG 转换为4PIN SWD，顺序为VCC -> SWDIO -> SWCLK -> GND

- PS：JTAG的PIN1 和 PIN2短接，这样J-LINK可以给目标板供电同时也给SW输出IO供电。

- Convert JTAG into 4PIN SWD in the order of vcc-> swdio-> swclk-> GND
PS: JTAG PIN1 and PIN2 short, so that J-Link can supply power to target board and SW output IO.

- 将J-Link 和目标板连接，4PIN SWD接入MC 控制板的SI01接口（注意，顺序为VCC/DIO/CLK/GND 要一一对应）。正确连接后MC板的MCU接入JLINK的3.3V电压LED2指示灯亮。

J-link is connected to the target board, and 4PIN SWD is connected to the SI01 interface of MC control board (note that VCC/DIO/CLK/GND should correspond to each other in order). After proper connection, the MCU of MC board is connected to LED2 indicator of JLINK's 3.3V voltage.

- PS：目标板切记不要接入AC220V，必须断开AC220V才允许接入SWD。因为SWD口是非隔离电路。Remember not to connect AC220V to the target board, AC220V must be disconnected before SWD can be connected. Because SWD port is not isolated circuit.

4、打开projet/ebldc_m0.uvprojx  Open projet/ebldc_m0.uvprojx

点击load按钮，程序下载，Click the load button to download the program,

下载完成  download completes

- 程序下载完成后将SWD口移除，一定要移除。移除之后才能将MC板子接入AC220V电源中。  Remove SWD port after downloading the program. Be sure to remove it. Only after removal can MC board be connected to AC220V power supply.

- 程序更新完成。Program update completed.